# HelixCluster Phase 2 ��� Console Compute Nodes (PS4/PS5)

Helix Development Team

2026-05-30

Table of Contents

# HelixCluster Phase 2 — Console Compute Nodes

## Executive Summary

### Project Context

HelixCluster Phase 1 established a distributed computing architecture that binds heterogeneous PCs and laptops into a single coherent compute block. Phase 2 extends this architecture to include jailbroken PlayStation 4, PS4 Pro, PS5, and PS5 Pro consoles as fully integrated worker nodes.

### Why Consoles?

The global installed base of PlayStation consoles exceeds 210 million units. Millions of these devices spend the majority of their time in REST mode or idle — representing an enormous reservoir of untapped compute power. At used market prices of $80-250 for PS4 and $400-500 for PS5, these devices deliver GPU compute at roughly half the cost per TFLOP of equivalent PC hardware.

### Console Hardware as Compute

| Console | CPU | GPU | RAM | Cost (Used) | GPU TFLOPS | $/TFLOP |
| --- | --- | --- | --- | --- | --- | --- |
| PS4 Base | 8x Jaguar 1.6GHz | GCN 1.84 TF | 8GB GDDR5 | $80-150 | 1.84 | $81 |
| PS4 Pro | 8x Jaguar 2.1GHz | GCN 4.20 TF | 8GB GDDR5 | $150-250 | 4.20 | $59 |
| PS5 | 8c/16t Zen2 3.5GHz | RDNA2 10.3 TF | 16GB GDDR6 | $400-500 | 10.3 | $49 |
| PS5 Pro | 8c/16t Zen2 3.85GHz | RDNA2+ ~33 TF | 16GB GDDR6+2GB | $550-700 | ~33 | ~$21 |

### Key Innovation: Linux on PlayStation

The foundational enabler for Phase 2 is Linux on PlayStation — a mature ecosystem for PS4 (kernels up to 6.15.4, Docker, full GPU acceleration) and a brand-new capability for PS5 (TheFlow’s ps5-linux, April 2026, Ubuntu 24.04 with GPU support). This transforms consoles from closed gaming appliances into general-purpose Linux servers.

### Unique Capabilities Consoles Bring

- GPU Compute at Half PC Cost — Discarded gaming hardware repurposed

- PS5 Custom I/O Decompressor — 8-9 GB/s hardware decompression (no PC equivalent)

- GDDR5/GDDR6 Unified Memory — 176-576 GB/s bandwidth vs DDR4’s 25-50 GB/s

- Disposable Node Model — At $80-250, failed nodes are replaced, not repaired

- Community Elastic Scaling — Users can donate idle console time

### Architecture Approach

Console nodes run a minimal Linux distribution with our Console Node Agent. They connect to the cluster via WireGuard mesh, register with TRUST_LEVEL=SEMI, and execute workloads through the same Vulkan Compute Backend used by PC nodes. The Console Adapter Layer handles console-specific concerns: thermal management, power monitoring, jailbreak state detection, and auto-exploit triggering.

### PS3 Exclusion

The PlayStation 3’s Cell Broadband Engine was thoroughly evaluated and excluded from Phase 2. Despite its historical significance (powering the Condor supercomputer at 500 TFLOPS for $2M), the Cell BE is obsolete: 192 GFLOPS vs a modern Ryzen 9’s 2.7 TFLOPS, extreme programming complexity, dead toolchain ecosystem, and a single Raspberry Pi 4 outperforms it in most metrics.

### Phase 2 Scope

- 24 new implementation tasks, ~176 hours (~4.5 weeks additional)

- Console Agent for PS4/PS5 Linux

- Vulkan Compute Backend (already universal — no console-specific GPU code needed)

- Semi-trusted security model with output verification

- Auto-exploit hardware integration for unattended operation

- PS5 Orbis OS native agent for custom I/O decompression

- llama.cpp AI inference on console GPU pool

- Console-aware scheduler plugin (thermal throttling awareness)

### Risk Summary

| Risk | Level | Mitigation |
| --- | --- | --- |
| Semi-tethered jailbreak | Medium | Auto-exploit hardware (ESP32/Luckfox), REST mode persistence |
| Thermal throttling | Medium | Thermal monitoring, workload backoff, fan control |
| Kernel compromised (inherent) | High (contained) | SEMI trust model, encrypted work units, output verification |
| AVX2 absence (PS4) | Low | Use SSE4.2/AVX fallback paths, Vulkan compute for GPU work |
| Hardware availability | Low | 117M+ PS4s, 93M+ PS5s in market |

# Chapter 2: Console Hardware Deep Dive

## 2.1 The Jailbreak Ecosystem

### GoldHen (PS4)

GoldHen is the primary homebrew enabler for PS4 consoles. It is a kernel-level payload that patches the PlayStation 4 operating system to enable:

- Homebrew execution — Run unsigned code

- Debug settings — Full system debugging access

- FTP server — Remote file transfer (port 2121)

- BinLoader — Load additional payloads (port 9090)

- Plugin system — Persistent plugins across sleeps

- Remote Package Installer — Install packages over network

- Rest Mode support — Jailbreak survives sleep/REST mode

GoldHen supports firmwares 5.05 through 9.60, with the 9.00 firmware being the sweet spot for stability and homebrew compatibility.

### etaHEN / UMTX2 (PS5)

For PS5, the jailbreak landscape is evolving rapidly:

- UMTX2 exploit (2024) — Full kernel exploit up to firmware 7.61

- etaHEN — Homebrew enabler supporting firmwares up to 10.01

- Userland Lua exploit — Up to firmware 10.40 on PS5 Pro (no kernel)

- kstuff payload — Enables PS4 FPKG backward compatibility

The PS5 Pro security co-processor (ARM Cortex-A53) independently handles PKG authentication, making native PS5 FPKG installation impossible even with full kernel access. However, this does NOT affect Linux/homebrew compute.

### Auto-Exploit Hardware

The primary operational challenge with console nodes is the semi-tethered jailbreak — a cold boot loses the jailbreak and requires re-exploitation. Two hardware solutions automate this:

- ESP32-based auto-exploit (~$5) — Programmable microcontroller that sends the USB exploit payload automatically when the console boots

- Luckfox MCU (~$8) — Similar approach with more sophisticated timing control

These devices connect to the PS4/PS5 USB port and act as a “jailbreak dongle” — the console boots, the MCU detects power-on, and automatically sends the exploit sequence. Combined with REST mode persistence (which preserves the jailbreak across days of sleep), the console is effectively always jailbroken.

## 2.2 PS4 Base (CUH-10xx/11xx) — Tier 3

### System-on-Chip: AMD Liverpool

The PS4 uses a semi-custom AMD APU codenamed “Liverpool,” built on a 28nm process. It combines:

- CPU: 8-core AMD Jaguar x86-64 at 1.6 GHz (up to 1.75 GHz boost)

- GPU: 18 Compute Units of AMD GCN 2.0 architecture

- Memory: 8 GB GDDR5 unified memory at 176 GB/s bandwidth

- Northbridge: Custom AMD with cache coherency

### CPU Deep Dive

The Jaguar CPU is a low-power x86-64 microarchitecture originally designed for tablets and mobile devices. Eight cores are arranged in two quad-core clusters, each with:

- 32KB L1 instruction cache + 32KB L1 data cache per core

- 2MB L2 cache per quad-core cluster (shared)

- Support for SSE4.1, SSE4.2, AVX, AES-NI

- NO AVX2 (significant limitation for some optimized code)

- Single-precision floating point throughput: ~115 GFLOPS aggregate

Performance context: Geekbench 4 single-core ~1400, multi-core ~7600. Approximately 5-6x slower than a modern Ryzen 5 desktop CPU. Comparable to an AMD Athlon 5370.

### GPU Deep Dive

The Liverpool GPU features:

- 18 Compute Units (CUs) = 1152 streaming processors

- AMD GCN 2.0 architecture (Sea Islands family)

- 1.84 TFLOPS single-precision compute

- 800 MHz base clock, 911 MHz boost

- 32 ROPs, 72 texture units

- Full hardware video encode/decode (H.264, AVC)

- 8 GB GDDR5 shared with CPU at 176 GB/s

### Linux on PS4 Base

Kernel support is excellent: - Linux 6.15.4 (latest) with full PS4 patches - AMDGPU kernel driver for GPU acceleration - Mesa 24.x with RADV (Radeon Vulkan) driver - Full Gigabit Ethernet support (Realtek RTL8153) - USB 3.0/2.0 full support - SATA SSD upgrade supported

## 2.3 PS4 Pro (CUH-70xx) — Tier 2 (RECOMMENDED)

### System-on-Chip: AMD Neo

The PS4 Pro uses an enhanced semi-custom APU codenamed “Neo”:

- CPU: 8-core AMD Jaguar x86-64 at 2.13 GHz (overclockable to ~2.6 GHz)

- GPU: 36 Compute Units of AMD GCN 4.0 (Polaris) architecture

- Memory: 8 GB GDDR5 (218 GB/s) + 1 GB DDR3 for OS

- Southbridge: Belize (faster than base PS4’s Bora)

### CPU Improvements

- 33% higher clock (2.13 GHz vs 1.6 GHz)

- Overclocking to 2.6 GHz is stable with good cooling

- Same Jaguar architecture — still no AVX2

- AES throughput: ~6.93 GB/s aggregate (vs ~5 GB/s on base)

### GPU Improvements

- 36 CUs (vs 18 on base) — exactly double

- GCN 4.0 Polaris architecture (vs GCN 2.0)

- 4.20 TFLOPS (vs 1.84) — 2.3x improvement

- 911 MHz clock

- Improved tessellation, delta color compression

- Hardware HEVC (H.265) decode support

### Key Advantages for Cluster Use

- Best cost/TFLOP ratio at ~$59/TFLOP

- KVM virtualization confirmed working (unique among consoles)

- WiFi AC for wireless cluster deployment

- USB 3.1 Gen1 x3 ports for storage expansion

- Overclockable CPU to 2.6 GHz for extra performance

## 2.4 PS5 (CFI-10xx/11xx) — Tier 1 (PREMIUM)

### System-on-Chip: AMD Oberon

The PS5 represents a generational leap — a true desktop-class SoC:

- CPU: 8-core/16-thread AMD Zen 2 at up to 3.5 GHz

- GPU: 36 CUs of AMD RDNA 2 at up to 2.23 GHz

- Memory: 16 GB GDDR6 at 448 GB/s

- Custom I/O Complex: Hardware decompression, DMA engine

- Process: TSMC 7nm

### CPU: Desktop-Class Zen 2

The PS5’s CPU is a custom Zen 2 design: - 8 cores, 16 threads — same as Ryzen 7 3700X - Variable frequency up to 3.5 GHz (lower than desktop’s 4.4 GHz) - 35% smaller FPU than desktop Zen 2 — slightly reduced FP performance - Full AVX2 support (unlike PS4’s Jaguar) - 4MB L3 cache per CCX (8MB total) - Estimated aggregate FP32: ~450 GFLOPS

Key limitation: The 35% smaller FPU means floating-point throughput is reduced compared to an equivalent desktop Ryzen. However, integer performance and general-purpose computing are unaffected.

### GPU: RDNA 2 with Ray Tracing

- 36 Compute Units of AMD RDNA 2

- 10.28 TFLOPS single-precision

- Hardware ray tracing acceleration

- Variable frequency up to 2.23 GHz

- Mesh shaders, variable rate shading support

- 16 GB GDDR6 shared with CPU

### Custom I/O Complex (Unique Advantage)

The PS5’s most unique hardware is its Custom I/O Complex:

- Kraken hardware decompressor: 8-9 GB/s compressed-to-raw

  - Equivalent to ~9 Zen 2 CPU cores of decompression work

  - Supports zlib, Kraken, and Oodle formats

- Direct Storage architecture: GPU can access SSD directly

- Custom DMA engine: Manages data flow without CPU involvement

- Custom flash controller: 5.5 GB/s raw, 8-9 GB/s compressed

CRITICAL: The Custom I/O Complex is NOT accessible from Linux. It is only available when running native Orbis OS code. For our cluster, this means: - Console nodes running Linux cannot use hardware decompression - We deploy a dual-mode agent: Linux primary + Orbis OS native fallback - The Orbis OS agent handles decompression-heavy workloads

### PS5 Linux Status (April 2026)

TheFlow’s ps5-linux project achieved: - Full Linux boot on PS5 Phat (firmwares 3.xx-4.xx) - Ubuntu 24.04 image available - 4K60 HDMI output - Full GPU acceleration via AMDGPU + Mesa - M.2 NVMe SSD support - Custom Ethernet driver (Gigabit) - CPU/GPU boost control utility

Target firmware for cluster PS5s: 3.xx-4.xx for best Linux compatibility, 4.51 for best overall jailbreak stability.

## 2.5 Comparative Analysis

### Console vs PC Cost Comparison

| Component | PS4 Pro Build | Equivalent PC | Savings |
| --- | --- | --- | --- |
| System | $200 (used) | — | — |
| CPU | 8x Jaguar 2.1GHz | Athlon 3000G ($50) | Included |
| GPU | 4.2 TFLOPS GCN | RX 570 4GB ($80 used) | Included |
| RAM | 8GB GDDR5 218GB/s | 8GB DDR4 ($25) | Included |
| SSD | 1TB SATA (upgrade) | 1TB SATA ($50) | — |
| PSU | Built-in | $30 | Included |
| Case | Built-in | $30 | Included |
| Total | $250 | $265+ | ~6% |

The savings increase dramatically when comparing GPU compute specifically:

| GPU Compute | Cost | TFLOPS | $/TFLOP |
| --- | --- | --- | --- |
| PS4 Pro (full system) | $200 | 4.2 | $48 |
| RX 580 8GB (GPU only) | $80 | 6.2 | $13 |
| RX 6600 (GPU only) | $180 | 8.9 | $20 |
| Full PC + RX 6600 | $600 | 8.9 | $67 |

When considering the complete system cost (not just GPU), the PS4 Pro is competitive, and the PS5 is significantly cheaper per TFLOP than a comparable PC build.

### Power Efficiency

| Device | TFLOPS | Watts (load) | TFLOPS/Watt |
| --- | --- | --- | --- |
| PS4 Base | 1.84 | 120W | 0.015 |
| PS4 Pro | 4.20 | 160W | 0.026 |
| PS5 | 10.28 | 200W | 0.051 |
| Desktop (RX 6600) | 8.93 | 250W | 0.036 |

The PS5 is the most power-efficient option, delivering 0.051 TFLOPS/Watt — better than a mid-range desktop GPU setup. # Chapter 3: Console Integration Architecture

## 3.1 High-Level Integration Overview

Console nodes are first-class citizens in the HelixCluster with specific adaptations. They connect through the same WireGuard mesh, register through the same Node Discovery service, and execute work through the same scheduler — but with a semi-trusted security model and console-specific adapters.

┌─────────────────────────────────────────────────────────────────────────────┐
│                    HELIXCLUSTER WITH CONSOLE NODES                           │
│                                                                             │
│   Control Plane (PC/Laptop)                    Console Worker Nodes         │
│   ┌─────────────────────────┐                  ┌─────────────────────────┐  │
│   │  API Gateway            │                  │  PS4 Pro (Tier 2)       │  │
│   │  Node Discovery         │◄── WireGuard ──►│  Linux 6.15 + Docker    │  │
│   │  Resource Scheduler     │     Mesh VPN     │  Console Node Agent     │  │
│   │  Session Manager        │                  │  Vulkan Compute         │  │
│   │  GPU Compute Engine     │◄─────────────────┤  llama.cpp inference    │  │
│   │  Health Monitor         │                  └─────────────────────────┘  │
│   │  LLM Brain              │                  ┌─────────────────────────┐  │
│   │  Build Service          │◄── WireGuard ──►│  PS5 (Tier 1)           │  │
│   │  Security Manager       │     Mesh VPN     │  Ubuntu 24.04           │  │
│   │  Backup Service         │                  │  Console Node Agent     │  │
│   └─────────────────────────┘                  │  Vulkan Compute         │  │
│          │                                     │  Orbis I/O Agent        │  │
│          │                                     └─────────────────────────┘  │
│          │                                                                   │
│          │    ┌────────────────────────────────────────────────────────┐    │
│          └───►│           SEMI-TRUSTED SECURITY MODEL                 │    │
│               │  • Encrypted work units only                          │    │
│               │  • All results verified (LLMsVerifier/redundant)      │    │
│               │  • No access to cluster state (etcd)                  │    │
│               │  • No sensitive data ever on console                  │    │
│               │  • Idempotent workloads only                          │    │
│               └────────────────────────────────────────────────────────┘    │
└─────────────────────────────────────────────────────────────────────────────┘

## 3.2 Console Node Agent Architecture

The Console Node Agent is a Go binary compiled for linux/amd64 that runs as a systemd service on the console’s Linux distribution.

### Component Diagram

┌──────────────────────────────────────────────────────────────┐
│              CONSOLE NODE AGENT (Go binary)                   │
│                                                              │
│  ┌────────────┐  ┌────────────┐  ┌────────────────────────┐ │
│  │   Core     │  │  Console   │  │     Workload Engine     │ │
│  │  Engine    │  │  Adapter   │  │                         │ │
│  │            │  │  Layer     │  │ ┌──────┐ ┌──────────┐  │ │
│  │ - Heartbeat│  │            │  │ │Batch │ │ GPU      │  │ │
│  │ - Resource │  │ - Thermal  │  │ │Worker│ │ Compute  │  │ │
│  │   Reporter │  │   Monitor  │  │ └──────┘ │ (Vulkan) │  │ │
│  │ - Task     │  │ - Power    │  │ ┌──────┐ └──────────┘  │ │
│  │   Receiver │  │   Manager  │  │ │AI    │ ┌──────────┐  │ │
│  │ - Result   │  │ - Jailbreak│  │ │Infer │ │ Storage  │  │ │
│  │   Reporter │  │   Monitor  │  │ │(LLM) │ │ (cache)  │  │ │
│  │ - WireGuard│  │ - Auto-    │  │ └──────┘ └──────────┘  │ │
│  │   Peer     │  │   Exploit  │  │ ┌──────┐               │ │
│  └────────────┘  │ - GPU      │  │ │Video │               │ │
│                  │   Monitor  │  │ │Trans │               │ │
│                  └────────────┘  │ └──────┘               │ │
│                                  └────────────────────────┘ │
└──────────────────────────────────────────────────────────────┘

### Core Engine

The Core Engine is identical to the PC Node Agent but with console-specific adaptations:

package agent

// ConsoleNodeAgent extends the base NodeAgent with console-specific capabilities
type ConsoleNodeAgent struct {
    *BaseNodeAgent  // Embedded: heartbeat, resource reporting, task execution
    
    ConsoleType     ConsoleType     // PS4_FAT, PS4_PRO, PS5, PS5_PRO
    Adapter         *ConsoleAdapter // Thermal, power, jailbreak management
    TrustLevel      TrustLevel      // Always SEMI for consoles
    
    // GPU compute (uses same Vulkan backend as PC)
    VulkanBackend   *vulkan.ComputeBackend
    
    // AI inference (llama.cpp subprocess)
    LLMEngine       *llama.InferenceEngine
}

func (a *ConsoleNodeAgent) RegisterWithCluster() error {
    node := &NodeRegistration{
        Type:        NODE_TYPE_CONSOLE,
        ConsoleType: a.ConsoleType,
        TrustLevel:  TRUST_SEMI,
        Resources:   a.scanConsoleResources(),
        Capabilities: []Capability{
            {Name: "gpu-vulkan-compute", Type: CAP_GPU},
            {Name: "ai-inference-llama", Type: CAP_AI},
            {Name: "batch-processing", Type: CAP_BATCH},
            {Name: "video-transcode", Type: CAP_VIDEO},
        },
    }
    return a.BaseNodeAgent.Register(node)
}

func (a *ConsoleNodeAgent) scanConsoleResources() *ConsoleNodeResources {
    return &ConsoleNodeResources{
        BaseResources: a.BaseNodeAgent.ScanResources(),
        ConsoleSpecific: ConsoleSpecificInfo{
            Model:           a.ConsoleType,
            Firmware:        a.Adapter.GetFirmwareVersion(),
            JailbreakVersion: a.Adapter.GetJailbreakVersion(),
            Thermal: ThermalState{
                CPUCurrentC:     a.Adapter.GetCPUTemp(),
                GPUCurrentC:     a.Adapter.GetGPUTemp(),
                FanSpeedPct:     a.Adapter.GetFanSpeed(),
                Throttling:      a.Adapter.IsThrottling(),
            },
            Power: PowerState{
                CurrentWatts: a.Adapter.GetPowerConsumption(),
            },
            GPU: GPUState{
                ClockMHz:        a.Adapter.GetGPUClock(),
                GPUTemperatureC: a.Adapter.GetGPUTemp(),
            },
        },
    }
}

### Console Adapter Layer

The Console Adapter is the unique component that handles console-specific hardware:

package console

// ConsoleAdapter manages console-specific hardware interfaces
type ConsoleAdapter struct {
    consoleType ConsoleType
    sysfsPath   string       // /sys/class/amdtep/ on PS4/PS5
}

// Thermal Management
func (a *ConsoleAdapter) GetCPUTemp() int {
    // Read from /sys/class/thermal/thermal_zone*/temp
    // PS4/PS5 expose thermal zones via standard Linux thermal framework
}

func (a *ConsoleAdapter) GetGPUTemp() int {
    // Read from AMDGPU sysfs: /sys/class/drm/card0/device/hwmon/temp1_input
}

func (a *ConsoleAdapter) SetFanSpeed(percent int) error {
    // Write to /sys/class/hwmon/hwmon*/pwm1
    // Range: 0-100%
}

func (a *ConsoleAdapter) IsThrottling() bool {
    cpuTemp := a.GetCPUTemp()
    gpuTemp := a.GetGPUTemp()
    // PS4 Pro throttles at ~85°C CPU, ~80°C GPU
    // PS5 throttles at ~90°C CPU, ~85°C GPU
    return cpuTemp > 85000 || gpuTemp > 80000 // millidegrees
}

// Power Management
func (a *ConsoleAdapter) GetPowerConsumption() float64 {
    // Read from /sys/class/power_supply/ if available
    // Fallback: estimate from CPU/GPU load + thermal state
}

// Jailbreak Management
func (a *ConsoleAdapter) IsJailbroken() bool {
    // Check if homebrew capabilities are available
    // On Linux: always true (kexec succeeded)
    // On Orbis: check for GoldHen/etaHEN presence
    return a.detectJailbreakMarker()
}

func (a *ConsoleAdapter) TriggerExploit() error {
    // Signal auto-exploit hardware to send payload
    // Via USB serial to ESP32/Luckfox
    // Or: trigger software exploit chain
}

// Auto-exploit via USB serial
func (a *ConsoleAdapter) initAutoExploit() error {
    port, err := serial.Open("/dev/ttyUSB0", &serial.Config{Baud: 115200})
    if err != nil {
        return fmt.Errorf("auto-exploit hardware not found: %w", err)
    }
    // Configure ESP32 for automatic exploit on console boot
    _, err = port.Write([]byte("CONFIG:AUTO_EXPLOIT=ON\n"))
    return err
}

## 3.3 Vulkan Compute Integration

### Universal GPU Backend

The most important architectural decision for Phase 2: no console-specific GPU code is needed. Our existing Vulkan Compute Backend works on all consoles without modification.

package vulkan

// ComputeBackend — same code runs on PC, PS4, PS4 Pro, PS5, PS5 Pro
type ComputeBackend struct {
    instance    vk.Instance
    device      vk.Device
    queue       vk.Queue
    queueFamily uint32
    memoryProps vk.PhysicalDeviceMemoryProperties
}

// Initialize discovers the GPU automatically
func NewComputeBackend() (*ComputeBackend, error) {
    // Vulkan enumerates all devices:
    // On PS4: AMD GCN Liverpool (radv)
    // On PS4 Pro: AMD GCN Polaris (radv)
    // On PS5: AMD RDNA2 Oberon (radv)
    // On PC: Whatever AMD/NVIDIA/Intel GPU is present
    // All use the SAME driver interface
}

// CompileShader compiles GLSL → SPIR-V → GPU binary
// SPIR-V is the universal intermediate representation
func (b *ComputeBackend) CompileShader(glslSource string) (*Shader, error) {
    // glslangValidator compiles GLSL to SPIR-V
    // SPIR-V is loaded by Vulkan on ANY GPU
}

### AI Inference: llama.cpp on Console

package llama

// InferenceEngine wraps llama.cpp for console AI workloads
type InferenceEngine struct {
    modelPath    string
    gpuLayers    int      // 99 = offload all to GPU
    port         int      // HTTP server port
    process      *os.Process
}

func (e *InferenceEngine) Start() error {
    // Launch llama.cpp server with Vulkan backend
    cmd := exec.Command("/opt/llama.cpp/llama-server",
        "-m", e.modelPath,
        "--gpu-layers", strconv.Itoa(e.gpuLayers),
        "--ctx-size", "8192",
        "--port", strconv.Itoa(e.port),
        "--host", "0.0.0.0",
    )
    // Set Vulkan device selection
    cmd.Env = append(os.Environ(),
        "GGML_VULKAN_DEVICE=0",  // Use first Vulkan GPU
    )
    return cmd.Start()
}

// Expected performance:
// PS4:    ~25 tok/s (3B model), ~9 tok/s (7B model)
// PS4 Pro: ~55 tok/s (3B model), ~20 tok/s (7B model)  
// PS5:     ~104 tok/s (3B model), ~38 tok/s (7B MoE)

## 3.4 Semi-Trusted Security Model

### Architecture

Console nodes operate at TRUST_LEVEL = SEMI. This is a deliberate security posture acknowledging that jailbroken consoles have fully compromised kernels.

┌────────────────────────────────────────────────────────────────┐
│               SEMI-TRUSTED NODE FLOW                           │
│                                                                │
│  1. Control Plane creates encrypted work unit                  │
│     (encrypted with console's public key)                      │
│                                                                │
│  2. Work unit sent to console via WireGuard                    │
│                                                                │
│  3. Console decrypts and executes                              │
│     (runs in sandbox/container)                                │
│                                                                │
│  4. Console signs result with its key                          │
│     (ed25519 signature)                                        │
│                                                                │
│  5. Result returned to control plane                           │
│                                                                │
│  6. Control plane verifies result:                             │
│     a) Cryptographic signature valid?                          │
│     b) LLMsVerifier checks output sanity?                      │
│     c) OR: Redundant compute on trusted node matches?          │
│                                                                │
│  7. Only verified results accepted into cluster state          │
│                                                                │
│  CONSOLE CANNOT:                                               │
│  • Read cluster state (etcd)                                   │
│  • Modify any cluster resource                                 │
│  • Access sensitive data                                       │
│  • Initiate any cluster operation                              │
│  • Communicate with other nodes directly                       │
└────────────────────────────────────────────────────────────────┘

### Implementation

package security

// SemiTrustedWorkUnit represents work sent to a console node
type SemiTrustedWorkUnit struct {
    ID          string            `json:"id"`
    Type        WorkType          `json:"type"`       // GPU_COMPUTE, AI_INFERENCE, BATCH
    EncryptedPayload []byte       `json:"payload"`    // Encrypted with console pubkey
    Environment map[string]string `json:"env"`        // Container environment
    Timeout     time.Duration     `json:"timeout"`
    VerifyMode  VerifyMode        `json:"verify_mode"` // LLM_VERIFY or REDUNDANT
}

type SemiTrustedResult struct {
    WorkUnitID  string            `json:"work_unit_id"`
    Output      []byte            `json:"output"`
    Signature   []byte            `json:"sig"`        // ed25519 signature
    Metrics     WorkMetrics       `json:"metrics"`    // Duration, GPU util, etc.
    ConsoleID   string            `json:"console_id"`
    Timestamp   time.Time         `json:"timestamp"`
}

func (s *SecurityManager) VerifyConsoleResult(result *SemiTrustedResult) error {
    // 1. Verify signature
    if !ed25519.Verify(consolePubkey, result.Output, result.Signature) {
        return ErrInvalidSignature
    }
    
    // 2. Check timestamp freshness (prevent replay)
    if time.Since(result.Timestamp) > 5*time.Minute {
        return ErrResultStale
    }
    
    // 3. Mode-specific verification
    switch result.VerifyMode {
    case LLM_VERIFY:
        // Use LLMsVerifier to check output sanity
        return s.llmVerifier.CheckOutput(result.Output)
    case REDUNDANT:
        // Compare with trusted node's result
        return s.compareWithTrusted(result)
    case TRIVIAL:
        // No verification needed (already trivial/known result)
        return nil
    }
    
    return nil
}

## 3.5 Scheduler Integration: Console-Aware Plugin

The scheduler needs to be aware of console-specific constraints:

package scheduler

// ConsoleAwarePlugin prevents scheduling inappropriate workloads on consoles
type ConsoleAwarePlugin struct {
    thermalThreshold int  // Celsius
}

func (p *ConsoleAwarePlugin) Filter(ctx context.Context, 
    state *framework.CycleState, pod *v1.Pod, 
    nodeInfo *framework.NodeInfo) *framework.Status {
    
    node := nodeInfo.Node()
    
    // Check if target is a console node
    if node.Labels["node-type"] != "console" {
        return framework.NewStatus(framework.Success) // Not a console, allow
    }
    
    // Console-specific filters:
    
    // 1. Don't schedule AVX2-required workloads on PS4
    if node.Labels["console-model"] == "ps4" || node.Labels["console-model"] == "ps4-pro" {
        if requiresAVX2(pod) {
            return framework.NewStatus(framework.Unschedulable, 
                "PS4 lacks AVX2 support")
        }
    }
    
    // 2. Don't schedule >8GB RAM workloads on PS4
    if node.Labels["console-tier"] == "3" && memoryRequest(pod) > 6*GiB {
        return framework.NewStatus(framework.Unschedulable,
            "PS4 has limited RAM")
    }
    
    // 3. Don't schedule sensitive data workloads on any console
    if containsSensitiveData(pod) {
        return framework.NewStatus(framework.Unschedulable,
            "Console nodes cannot access sensitive data")
    }
    
    // 4. Check thermal state
    if isConsoleOverheating(node) {
        return framework.NewStatus(framework.Unschedulable,
            "Console thermal throttling")
    }
    
    // 5. Console nodes only get idempotent workloads
    if !isIdempotent(pod) {
        return framework.NewStatus(framework.Unschedulable,
            "Console nodes require idempotent workloads")
    }
    
    return framework.NewStatus(framework.Success)
}

func (p *ConsoleAwarePlugin) Score(ctx context.Context,
    state *framework.CycleState, pod *v1.Pod,
    nodeInfo *framework.NodeInfo) (int64, *framework.Status) {
    
    node := nodeInfo.Node()
    if node.Labels["node-type"] != "console" {
        return 0, nil // No score modification for non-consoles
    }
    
    score := int64(100)
    
    // Penalize overheating consoles
    cpuTemp, _ := getCPUTemp(node)
    if cpuTemp > 80 {
        score -= int64(cpuTemp - 80) * 5  // -5 points per degree over 80
    }
    
    // Bonus for consoles with good thermal headroom
    if cpuTemp < 60 {
        score += 20
    }
    
    // Prefer PS5 for GPU-intensive workloads
    if isGPUWorkload(pod) && node.Labels["console-tier"] == "1" {
        score += 50
    }
    
    return score, nil
}

## 3.6 Health Monitoring: Console-Specific Metrics

# Console-specific Prometheus metrics
# These are collected by the Console Adapter and exposed by the node agent

# Temperature metrics
console_cpu_temperature_celsius{node_id="ps4-pro-001"} 72
console_gpu_temperature_celsius{node_id="ps4-pro-001"} 68
console_fan_speed_percent{node_id="ps4-pro-001"} 45

# Power metrics  
console_power_consumption_watts{node_id="ps4-pro-001"} 142.5
console_power_daily_kwh{node_id="ps4-pro-001"} 2.8

# GPU metrics (console-specific)
console_gpu_clock_mhz{node_id="ps4-pro-001"} 911
console_gpu_vram_used_bytes{node_id="ps4-pro-001"} 2147483648
console_gpu_throttling{node_id="ps4-pro-001"} 0

# Jailbreak metrics
console_jailbreak_active{node_id="ps4-pro-001"} 1
console_jailbreak_version{node_id="ps4-pro-001", version="2.4b"} 1
console_linux_uptime_seconds{node_id="ps4-pro-001"} 172800

# Storage health
console_ssd_health_percent{node_id="ps4-pro-001"} 94
console_ssd_wear_level{node_id="ps4-pro-001"} 6
console_ssd_power_on_hours{node_id="ps4-pro-001"} 8760

# Thermal throttling alerts
- alert: ConsoleThermalThrottling
  expr: console_cpu_temperature_celsius > 85 or console_gpu_temperature_celsius > 80
  for: 2m
  labels:
    severity: warning
  annotations:
    summary: "Console {{ $labels.node_id }} is thermal throttling"

- alert: ConsoleJailbreakLost
  expr: console_jailbreak_active == 0
  for: 1m
  labels:
    severity: critical
  annotations:
    summary: "Console {{ $labels.node_id }} lost jailbreak"

## 3.7 Auto-Exploit Hardware Integration

For unattended console cluster nodes, auto-exploit hardware automates the jailbreak process:

### ESP32 Auto-Exploit Setup

// ESP32 firmware for automatic PS4/PS5 jailbreak
// Connects to console USB port, sends exploit on boot detection

#include <USB.h>
#include <PS4Exploit.h>  // Custom exploit payloads

const int CONSOLE_POWER_SENSE_PIN = 4;  // GPIO connected to console power LED
bool consolePowered = false;

void setup() {
    pinMode(CONSOLE_POWER_SENSE_PIN, INPUT);
    Serial.begin(115200);
    
    // Load exploit payload for target firmware
    loadPayload("goldhen_2.4b_900.bin");
}

void loop() {
    bool currentPower = digitalRead(CONSOLE_POWER_SENSE_PIN);
    
    if (currentPower && !consolePowered) {
        // Console just powered on — send exploit
        Serial.println("Console boot detected, sending exploit...");
        delay(5000);  // Wait for USB stack initialization
        sendExploitPayload();
        consolePowered = true;
    }
    
    if (!currentPower && consolePowered) {
        // Console powered off
        consolePowered = false;
    }
    
    delay(1000);
}

### Provisioning Integration

The setup wizard for console nodes includes auto-exploit hardware configuration:

$ htmux cluster add-console --auto-exploit

[Auto-Exploit Setup]
1. Connect ESP32 to console's front USB port
2. Connect ESP32 to cluster management network (WiFi)
3. Flashing auto-exploit firmware...
   [████████████████] 100%
4. Configuring for firmware 9.00 (detected)...
5. Testing auto-exploit cycle...
   Power off → Power on → Exploit sent ✓ → GoldHen loaded ✓
6. Console will now auto-jailbreak on every boot.

# Chapter 4: Phase 2 Implementation Plan & Console Setup

## 4.1 Phase 2 Task Breakdown

HelixCluster Phase 2 adds 24 new tasks (~176 hours, ~4.5 weeks) to the existing Phase 0-8 plan. These tasks are distributed across all phases, with the heaviest concentration in Phase 0 (foundations) and Phase 1 (infrastructure).

### Console-Specific Task Matrix

| Phase | Task ID | Description | Hours | Priority | Skill |
| --- | --- | --- | --- | --- | --- |
| 0 | C-0.1 | Console Agent Go project scaffolding | 8h | P0 | GO |
| 0 | C-0.2 | ConsoleAdapter interface definition | 4h | P0 | GO |
| 0 | C-0.3 | Thermal/power monitoring via sysfs/hwmon | 8h | P0 | GO |
| 0 | C-0.4 | Jailbreak detection library | 8h | P0 | GO |
| 0 | C-0.5 | Auto-exploit ESP32 firmware (C++) | 16h | P1 | C |
| 0 | C-0.6 | Console capability scanner | 4h | P0 | GO |
| 0 | C-0.7 | PS5 Orbis I/O Agent (native) | 16h | P2 | C |
| 1 | C-1.1 | Console node registration (SEMI trust) | 4h | P0 | GO |
| 1 | C-1.2 | Console heartbeat with thermal metrics | 4h | P0 | GO |
| 1 | C-1.3 | WireGuard kernel module for PS4/PS5 Linux | 4h | P0 | GO |
| 1 | C-1.4 | ZeroMQ lightweight client for PS4 | 4h | P0 | GO |
| 1 | C-1.5 | gRPC client for PS5 | 4h | P0 | GO |
| 2 | C-2.1 | Vulkan Compute Backend validation on PS4/PS5 | 8h | P0 | C |
| 2 | C-2.2 | llama.cpp Vulkan integration for consoles | 8h | P0 | C |
| 2 | C-2.3 | Console-specific ClassAds expressions | 4h | P0 | GO |
| 2 | C-2.4 | ConsoleAware scheduler plugin | 8h | P0 | GO |
| 3 | C-3.1 | Minimal PTY session backend for consoles | 8h | P1 | GO |
| 4 | C-4.1 | AOSP distcc worker on PS4 | 8h | P1 | GO |
| 4 | C-4.2 | AOSP distcc + GPU worker on PS5 | 8h | P1 | GO |
| 5 | C-5.1 | AI inference agent (llama.cpp server) | 8h | P1 | GO |
| 7 | C-7.1 | Console chaos tests (power loss, thermal) | 8h | P0 | QA |
| 7 | C-7.2 | SEMI trust model verification testing | 8h | P0 | QA |
| 8 | C-8.1 | Console setup wizard (htmux add-console) | 8h | P0 | GO |
| 8 | C-8.2 | Auto-exploit hardware provisioning | 8h | P1 | GO |

### Critical Path for Phase 2

C-0.1 (scaffold) → C-0.2 (adapter) → C-0.3 (thermal) → C-0.4 (jailbreak)
     │                                                   │
     ▼                                                   ▼
C-0.6 (scanner)                                  C-1.1 (registration)
     │                                                   │
     ▼                                                   ▼
C-1.2 (heartbeat) → C-1.3 (WireGuard) → C-1.4/C-1.5 (messaging)
     │                                                   │
     ▼                                                   ▼
C-2.3 (ClassAds) → C-2.4 (scheduler plugin) ← C-2.1 (Vulkan test)
     │
     ▼
C-2.2 (llama.cpp) → C-5.1 (AI inference)
     │
     ▼
C-7.1 (chaos) → C-7.2 (verification) → C-8.1 (setup wizard) → C-8.2 (auto-exploit)

### Integration Points with Existing Components

| Existing Component | Console Integration | Effort |
| --- | --- | --- |
| Node Discovery | Add CONSOLE node type, SEMI trust level | Low |
| Resource Scheduler | ConsoleAware plugin (thermal, AVX2, RAM filters) | Medium |
| GPU Compute Engine | No changes — Vulkan backend is universal | None |
| Health Monitor | Add console-specific metrics (thermal, power, SSD) | Medium |
| LLM Brain | Console AI inference pool for parallel agents | Low |
| Build Service | Console distcc workers for AOSP | Low |
| Security Manager | SEMI trust model, encrypted work units | Medium |
| Session Manager | Minimal PTY for console nodes (no migration) | Low |

## 4.2 Console Setup Wizard

The console setup wizard is invoked via htmux cluster add-console and automates the entire provisioning process.

### Phase 1: Discovery

$ htmux cluster add-console --discover

Scanning local network for PlayStation consoles...

[DISCOVERED CONSOLES]
┌────┬──────────┬─────────────┬──────────────────┬────────────┬────────┐
│ ## │ Model    │ IP Address  │ MAC Address      │ Firmware   │ Status │
├────┼──────────┼─────────────┼──────────────────┼────────────┼────────┤
│ 01 │ PS4 Pro  │ 192.168.1.45│ A4:17:31:XX:XX:XX│ 9.00      │ JB ✓   │
│ 02 │ PS4 Fat  │ 192.168.1.47│ A4:17:31:XX:XX:XX│ 11.00     │ No JB  │
│ 03 │ PS5      │ 192.168.1.50│ 88:C9:E8:XX:XX:XX│ 4.51      │ JB ✓   │
└────┴──────────┴─────────────┴──────────────────┴────────────┴────────┘

Note: PS4 at 192.168.1.47 (firmware 11.00) cannot be jailbroken.
       Only firmwares ≤9.00 (PS4) or ≤4.51 (PS5) are exploitable.

Select consoles to add (comma-separated): 1,3

### Phase 2: Jailbreak

[PS4 Pro at 192.168.1.45]
Firmware: 9.00 ✓ (GoldHen-compatible)
Preparing jailbreak payload...

  [████████████████████] 100%
  GoldHen v2.4b loaded successfully
  Debug settings enabled
  FTP server active on port 2121
  BinLoader active on port 9090

[PS5 at 192.168.1.50]
Firmware: 4.51 ✓ (etaHEN-compatible)
Preparing jailbreak payload...

  [████████████████████] 100%
  etaHEN loaded successfully
  Homebrew enabled
  FTP server active on port 2121

### Phase 3: Linux Installation

[Installing Linux on PS4 Pro]
Downloading psxitarch v3 (kernel 6.15.4)...
  [████████████████████] 100% 1.2 GB downloaded

Preparing USB drive /dev/sdb...
Writing Linux payload...
  [████████████████████] 100%

Booting Linux via kexec...
  [████████████████████] 100%
  Linux 6.15.4-ps4 booted successfully
  8 CPU cores detected
  AMDGPU loaded (36 CUs)
  6.85 GB RAM available
  Gigabit Ethernet: UP (1000 Mbps)

[Installing Linux on PS5]
Downloading Ubuntu 24.04 for PS5...
  [████████████████████] 100% 2.1 GB downloaded

Writing to USB drive...
Booting via ps5-linux-loader...
  [████████████████████] 100%
  Ubuntu 24.04 booted successfully
  16 CPU threads detected (Zen 2)
  AMDGPU loaded (RDNA2, 36 CUs)
  12.5 GB RAM available
  Gigabit Ethernet: UP (1000 Mbps)
  M.2 SSD: detected

### Phase 4: Agent Installation

[Installing HelixCluster Console Agent]

Downloading console-agent-linux-amd64...
  [████████████████████] 100%

Installing systemd service...
  Creating user: helix (no sudo)
  Installing binary: /opt/helix/bin/console-agent
  Creating service: /etc/systemd/system/helix-console.service
  Enabling auto-start: ✓

Configuring agent...
  Control plane: auto-discovered at 192.168.1.10:8443
  WireGuard mesh: generating keys...
  Node labels: tier=2,model=ps4-pro

Starting agent...
  [████████████████████] 100%
  Agent running, PID 1847

### Phase 5: Cluster Registration

[Registering with HelixCluster]

PS4 Pro (192.168.1.45):
  Node ID: c4f8e2d1-7a3b-4c5d-9e0f-1a2b3c4d5e6f
  Trust Level: SEMI
  Tier: 2 (Standard)
  WireGuard IP: 100.64.2.15
  ┌─────────────────────────────────────────┐
  │  CAPABILITIES                           │
  │  ✓ gpu-vulkan-compute (GCN 4.0, 4.2TF) │
  │  ✓ ai-inference-llama (55 tok/s 3B)    │
  │  ✓ batch-processing (8x Jaguar)        │
  │  ✓ video-transcode (GPU shader)        │
  └─────────────────────────────────────────┘

PS5 (192.168.1.50):
  Node ID: d5e9f3g2-8b4c-5d6e-0a1b-2c3d4e5f6a7b
  Trust Level: SEMI
  Tier: 1 (Premium)
  WireGuard IP: 100.64.2.16
  ┌─────────────────────────────────────────┐
  │  CAPABILITIES                           │
  │  ✓ gpu-vulkan-compute (RDNA2, 10.3TF)  │
  │  ✓ ai-inference-llama (104 tok/s 3B)   │
  │  ✓ batch-processing (Zen2 8c/16t)      │
  │  ✓ video-transcode (GPU shader)        │
  │  ✓ hardware-decompress (Kraken, Orbis) │
  └─────────────────────────────────────────┘

═══════════════════════════════════════════════════════════════
  ✓ 2 console nodes successfully added to HelixCluster!
═══════════════════════════════════════════════════════════════

Cluster GPU TFLOPS: +14.5 (4.2 from PS4 Pro, 10.3 from PS5)
Cluster CPU cores:   +24 (8 from PS4 Pro, 16 from PS5)
Cluster RAM:         +20.35 GB

View status: htmux cluster status

## 4.3 Auto-Exploit Hardware Kit

### Bill of Materials

| Component | Model | Cost | Purpose |
| --- | --- | --- | --- |
| ESP32-S2 DevKit | NodeMCU-32-S2 | $4.50 | Auto-exploit MCU |
| USB-A Cable | 1ft right-angle | $2.00 | Connect to console |
| 3D Printed Case | Custom design | $1.00 | Enclosure |
| JST Connector | 2-pin | $0.50 | Power sense wire |
| Total per kit |  | ~$8 |  |

### Assembly

ESP32-S2 Wiring:
┌────────────────────────────────────┐
│         ESP32-S2 NodeMCU           │
│                                    │
│  GPIO 4 ──────► Power sense (opt) │
│  GPIO 19/20 ──► USB D-/D+         │
│  5V/GND ──────► USB power         │
│                                    │
│  USB-C (power/programming)         │
└────────────────────────────────────┘
        │
        ▼
┌────────────────────────────────────┐
│     PS4/PS5 Front USB Port         │
│                                    │
│  [USB-A] ←────── ESP32-S2         │
│  [USB-A] ←────── Other devices    │
│  [USB-C]                          │
└────────────────────────────────────┘

### Firmware Flashing

# Via htmux CLI
$ htmux cluster provision-auto-exploit --device /dev/ttyUSB0 --console ps4-pro-001

Flashing auto-exploit firmware to ESP32...
  Chip: ESP32-S2 (revision 0)
  Flash size: 4MB
  [████████████████████] 100%

Configuring:
  Target firmware: 9.00 (from console registration)
  Exploit type: GoldHen (USB method)
  Auto-trigger: ON (power sense)
  LED indicator: ON

Testing:
  Simulating console boot...
  Exploit payload sent ✓
  Expected GoldHen load: ~8 seconds
  
✓ Auto-exploit hardware provisioned for ps4-pro-001

## 4.4 Community Console Donation Model

A unique capability enabled by the semi-trusted model: community members can donate idle console time.

┌─────────────────────────────────────────────────────────────────┐
│              COMMUNITY CONSOLE DONATION                          │
│                                                                  │
│  [Community Member]                    [HelixCluster]           │
│  ┌──────────────────┐                  ┌──────────────────┐     │
│  │ "I have a PS4    │                  │ "Accepting       │     │
│  │  that's idle     │ ── Register ───► │  console nodes   │     │
│  │  22 hours/day"   │                  │  for AI inference │    │
│  └──────────────────┘                  └──────────────────┘     │
│         │                                      │                │
│         │  htmux cluster donate-console       │                │
│         │  --hours 22:00-06:00                │                │
│         │  --workload-types ai-inference      │                │
│         │                                      │                │
│         ▼                                      ▼                │
│  ┌──────────────────────────────────────────────────────┐      │
│  │  TIME-SHARED CONSOLE NODE                            │      │
│  │                                                      │      │
│  │  06:00 - 22:00 │ Gaming mode (console owner uses it) │      │
│  │  22:00 - 06:00 │ Cluster mode (AI inference, batch)  │      │
│  │                                                      │      │
│  │  Work units are:                                     │      │
│  │  • Encrypted (console never sees data)               │      │
│  │  • Verified (results checked by trusted nodes)       │      │
│  │  • Interruptible (gaming takes priority)             │      │
│  │  • Compensated (owner receives compute credits)      │      │
│  └──────────────────────────────────────────────────────┘      │
└─────────────────────────────────────────────────────────────────┘

## 4.5 Performance Validation Targets

### Acceptance Criteria for Phase 2

| Test | Target | Measurement |
| --- | --- | --- |
| PS4 Pro Vulkan compute | ≥3.5 TFLOPS sustained | clpeak benchmark |
| PS5 Vulkan compute | ≥8.5 TFLOPS sustained | clpeak benchmark |
| llama.cpp PS4 Pro (3B) | ≥50 tok/s | llama-bench |
| llama.cpp PS5 (3B) | ≥100 tok/s | llama-bench |
| AOSP build contribution | ≥8 parallel jobs | distcc monitor |
| Network throughput | ≥850 Mbps | iperf3 |
| Thermal stability (24h) | <80°C CPU sustained | stress-ng + monitoring |
| Jailbreak persistence | 30+ days in REST mode | Longevity test |
| Auto-exploit reliability | 99%+ success rate | 100 boot cycles |
| Console agent memory | <128 MB RAM | ps/aux measurement |
| Workload verification | 100% accuracy | Redundant compute check |
| Cluster integration | Same APIs as PC nodes | End-to-end test |